# Assignment 4 — Implementation Plan

Team 14 | Option 2b: Simulation Experiments | N1/N2 Network

# 1. Goal

Analyse the vulnerability and criticality of Bangladesh's N1/N2 transport network using simulation experiments. Build on the A3 Mesa/NetworkX model. Answer: Which bridges should Bangladesh prioritise for maintenance and investment?

# 2. Key Definitions

## Criticality

Definition: The importance of a bridge to overall network performance. Measured by how much average travel time increases when that bridge is removed (Δ travel time).

Source: Jafino, Kwakkel & Verbraeck (2020) — "change in total travel cost" metric.

## Vulnerability

Definition: How likely a bridge is to fail AND how bad the consequences are. Vulnerability = P(failure) × Expected delay caused.

Sources: Berdica (2002); Mattsson & Jenelius (2015).

Key insight: The disaster data tells us WHO is likely to fail (probability). The simulation tells us HOW BAD it is when they do (consequence). You need both.

# 3. Data-Driven Failure Probabilities

Each bridge gets a unique failure probability based on two factors:

## Factor 1: Bridge Condition (from BMMS)

| Condition | Base Probability | Meaning |
| --- | --- | --- |
| A (Good) | 0.02 | Unlikely to fail |
| B (Fair) | 0.05 | Low risk |
| C (Poor) | 0.10 | Moderate risk |
| D (Very Poor) | 0.20 | High risk |

## Factor 2: Disaster Exposure (from team member's data)

| Hazard Type | Data Source | Multiplier Examples |
| --- | --- | --- |
| Earthquake | Seismic zone map | 1.0× / 1.5× / 2.0× |
| Rainfall | Precipitation data | 1.0× / 1.3× / 1.8× |
| Flood risk | Flood zone maps | 1.0× / 1.5× / 2.5× |
| Cyclone | Cyclone path data | 1.0× / 1.5× |

Formula: P(failure) = min( Base_condition × Hazard_multiplier, 1.0 )

Example: A condition-C bridge (base 0.10) in a high flood zone (2.0×) gets P = 0.20. The same condition-C bridge in a safe area stays at P = 0.10.

# 4. How Bridge Failure Works

We use one failure mode — degraded (same as A3, but with better probabilities).

At t=0: Each bridge gets a coin flip using its unique P(failure). If it fails → broken for the entire run (causes delays). If it doesn't → works normally.

When a truck reaches a broken bridge: It waits (45–240 min depending on bridge length), then continues. Same as A3.

Bridge removal (for criticality testing) is a separate experiment (Round 2), not a failure mode.

# 5. Other Model Changes from A3

Per-source truck generation: Use RMMS AADT data so busy roads (N1 near Dhaka) generate more trucks than quiet side roads. Replaces the fixed "1 truck every 5 min" everywhere.

Per-bridge traffic counter: Count how many trucks cross each bridge. Needed for Round 1 to identify the most-used bridges.

Total new code: ~60 lines on top of the existing A3 model.

# 6. Experiment Design (4 Iterative Rounds)

Each round builds on the previous — this is the "explorative and iterative process" the rubric asks for.

| Round | Question | What We Do | What We Learn |
| --- | --- | --- | --- |
| 1: Baseline | What does normal look like? | All bridges working, calibrated traffic. 10 replications. | Which bridges carry most trucks → informs Round 2 |
| 2: Bridge Removal (Criticality) | What if this bridge is gone? | Remove top bridges one-by-one, trucks reroute. 10 reps each. | Two metrics: Δ travel time (efficiency loss) + stranded trucks (connectivity loss) |
| 3: Stochastic Failure (Vulnerability) | What happens under disaster conditions? | Disaster-weighted probabilities, bridges degrade. Multiple scenarios × 10 reps. | Vulnerability score = P(failure) × avg delay caused |
| 4: Combined Analysis | Which bridges need attention first? | No new runs. Cross-reference Round 2 + Round 3 data. | Criticality × Vulnerability scatter plot → top priority bridges |

## Round 2: Two Criticality Metrics (Reported Separately)

We measure two separate criticality metrics — we do NOT merge them into one formula because they capture fundamentally different things:

| Metric | What It Captures | Unit |
| --- | --- | --- |
| Δ average travel time (primary) | How much longer trips take — measures efficiency loss | Minutes |
| Stranded trucks (supplementary) | How many trucks can't reach destination — measures connectivity loss | Count |

How to present: Rank bridges by Δ travel time (primary metric), show stranded trucks as an additional column. In the scatter plot, use dot size or color to indicate stranding.

Example results table:

| Rank | Bridge | Road | Δ Avg Travel Time | Stranded Trucks |
| --- | --- | --- | --- | --- |
| 1 | Bridge X | N1 | +85 min | 12 |
| 2 | Bridge Y | N2 | +62 min | 0 |
| 3 | Bridge Z | N1 | +45 min | 8 |

Insight: Bridge X causes delays AND disconnects the network. Bridge Y causes delays but alternatives exist (0 stranded). Bridge Z has moderate delays but also causes disconnection — arguably more critical than the delays alone suggest.

## Round 3 Scenarios

Or like what Brenda said (need to decide)

| Scenario | Description |
| --- | --- |
| 0: Baseline | No failures (same as Round 1) |
| 1: Monsoon | High rainfall + flood multipliers active |
| 2: Cyclone | Cyclone multiplier active for coastal bridges |
| 3: Earthquake | Earthquake multiplier active for seismic zones |
| 4: Worst-case | All hazard multipliers at maximum |

# 7. Key Visualisations

Geographic map — road segments colored by truck volume (Round 1)

Geographic map — bridges colored by failure probability (condition × disaster)

Bar chart — Top-10 critical bridges by Δ travel time, with stranded trucks column (Round 2)

Bar chart — Top-10 vulnerable bridges by vulnerability score (Round 3)

Scatter plot — Criticality vs Vulnerability, dot size/color = stranded trucks (Round 4)

Boxplots — Travel time distributions across scenarios

# 8. Suggested Team Division

| Member | Responsibility |
| --- | --- |
| A | Literature review, definitions, report writing & editing |
| B | Data preparation: merge disaster + bridge + traffic data, compute failure probabilities |
| C | Model adaptation: per-bridge probs, bridge removal, per-source generation, data collectors |
| D | Run experiments (Rounds 1-4), analysis notebooks, all visualisations |

# 9. Data Requirements

| Data | Source | Status |
| --- | --- | --- |
| Road network + bridge conditions | A3 model (network_data.csv, BMMS) | ✅ Have it |
| Truck AADT per road | RMMS dataset (course materials) | ⚠️ Zhaf has it |
| Disaster data (earthquake, flood, rain, cyclone) | Team member's data | ⚠️ Brenda has it |
| GIS layers | A3 data/gis/ folder | ✅ Have it |